# Emotion Classifier Using Verilog

Final Project Report

Date: 27 November 2025

# 1. Executive Summary

This project implements a hardware/software co-design for real-time emotion classification. It combines a high-performance face detector implemented in Verilog (for FPGA deployment) with a flexible emotion classification neural network running in Python. The system leverages the Verilog Procedural Interface (VPI) to enable seamless communication between the hardware simulation and the software model.

# 2. System Architecture

The system consists of three main layers: the Verilog Hardware Layer, the VPI Interface Layer, and the Python AI Server Layer.

## 2.1 Architecture Diagram

┌─────────────────────────────────────────────────────────────────────────────┐
│                           VERILOG HARDWARE LAYER                            │
│                         (Icarus Verilog Simulator)                          │
├─────────────────────────────────────────────────────────────────────────────┤
│  ┌───────────────────────────────────────────────────────────────────────┐  │
│  │                        Face Detector Module                           │  │
│  │                      (Haar Cascade Algorithm)                         │  │
│  └───────────────────────────────────────────────────────────────────────┘  │
│                                     │                                       │
└─────────────────────────────────────┼─────────────────────────────────────┘
                                      ▼
┌─────────────────────────────────────────────────────────────────────────────┐
│                         VPI C INTERFACE LAYER                               │
│                    (verilog_python_interface.c)                             │
└─────────────────────────────────────┼─────────────────────────────────────┘
                                      ▼
┌─────────────────────────────────────────────────────────────────────────────┐
│                         PYTHON AI SERVER LAYER                              │
│                          (emotion_server.py)                                │
│  ┌───────────────────────────────────────────────────────────────────────┐  │
│  │                    Mini-Xception Neural Network                       │  │
│  └───────────────────────────────────────────────────────────────────────┘  │
└─────────────────────────────────────────────────────────────────────────────┘

## 2.2 Key Components

Face Detector (Verilog): Implements the Viola-Jones Haar Cascade algorithm. It processes 64x64 grayscale images using integral images and a cascade of weak classifiers to detect faces.

VPI Interface (C/C++): Acts as a bridge, extracting the Region of Interest (ROI) from the Verilog memory and transmitting it over a TCP socket.

Emotion Server (Python): Hosts a Mini-Xception Convolutional Neural Network (CNN). It receives the image data, preprocesses it (resize to 48x48, normalize), and classifies it into one of 7 emotions (Angry, Disgust, Fear, Happy, Sad, Surprise, Neutral).

# 3. Work Accomplished

Implemented the full Haar Cascade detection pipeline in Verilog (Integral Image, Feature Calculation, Strong/Weak Classifiers).

Developed a Python TCP server to host the emotion classification model.

Created a C-based VPI module to facilitate data transfer between Verilog simulation and Python.

Designed a testing pipeline (`test_pipeline.py`) that automates image preparation, simulation, and result verification.

Provided Tcl scripts for seamless integration with Xilinx Vivado for FPGA synthesis.

# 4. Findings & Results

The co-simulation successfully demonstrates the feasibility of offloading the computationally intensive face detection task to hardware while keeping the complex deep learning inference in software. This approach balances performance with flexibility.

## 4.1 Latency Analysis (Estimated)

| Stage | Time (ms) | Percentage |
| --- | --- | --- |
| Face Detection (HW) | 5.0 | 50% |
| Data Transfer (VPI/TCP) | 1.0 | 10% |
| Emotion Inference (SW) | 3.0 | 30% |
| Overhead | 1.0 | 10% |

# 5. Visualizations & Graphs

To generate the visual artifacts for this project, follow these instructions:

## 5.1 Architecture Diagram

Run the visualization script to see the detailed ASCII block diagram:

python3 verilog_face_detector/visualize_architecture.py

## 5.2 Simulation Waveforms

To view the signal-level behavior of the hardware face detector:

Navigate to the 'verilog_face_detector' directory.

Run 'make run-cosim' to generate the waveform dump.

Run 'make wave' to open GTKWave with the pre-configured 'sim/waveform.gtkw' file.